Name, formula:		Sodium Chloride, NaCl

Trade name:			Marine Salt De Icing Grade

Typical chemical analysis:	NaCl					≥99.0%

Sulphate SO4				<0.5%

Insolubles				≤0.15%

H20 					<4.0% (loss of mass at 110c)

Typical screen analysis:	Above 8mm     			< 1%

Passing 6.3mm 			≤ 5%

Passing 0.315mm 			≤ 5%

Treated with Anti-caking treatment 50 mg/kg

Forms of delivery:

Bulk:				By pneumatic tanker or tipper lorry

Available in 5kg, 10kg, 15kg, 20kg, 25kg, 850kg, 1000kg and Bulk.

This specification is given as an indication.  Being a natural product, variations can occur.

This product complies to BS 3247 2011+A1 2016