Supplier: 	J C Peacock & company Ltd. t/a Peacock Salt

North Harbour, Ayr, KA8 8AE

Tele:01292292000

Fax: 01292292001

Email: info@peacocksalt.co.uk

Composition:			50wt% Potassium Formate solution

Corrosion Inhibitors:		<0.5 wt%

Odour:				None

Surface Active Compounds:	None

| Viscosifiers: None Temperature | Viscosity (cP) |
| --- | --- |
| -30 | 25 |
| -20 | 12 |
| -10 | 7.2 |
| 0 | 5.0 |
| 10 | 3.7 |
| 20 | 2.9 |
| 30 | 2.1 |
| 40 | 1.8 |

Appearance:   			Clear and Colourless liquid

Density at 20° C		1330-1370kg/m3

Viscosity at 0°C		5 cP max

25°C		3 cP max

Flash Point(boiling point)	>100°C

Freezing Point:			less than -50°C

Miscibility with Water		Complete

pH				Typical 10.9-11.4

pH 10% w/w dest. Water	Typical 10.5

Materials compatible with Viaform Liquid.

| Metals | Polymers, composite materials and Rubbers |
| --- | --- |
| Stainless steel | Polyethylene |
| Aluminum Alloys | Ploycarbonate |
| Magnesium alloys | Acrilic |
| Titanium | Polychiroprene |
| Cadmium plated Steel | Polytetraflouoethene |
| Copper | Vulcanized Ethylene-propylene |
| Carbon Steel | Painted surfaces |
|  | Bitumen |
|  | Glass |
|  | Reinforced polyester fibreglass |
|  | Silicone |
|  | Polymethacylate |
|  | Concrete |
|  | Rubber Std. (N/R 1) (H) (FPM/1) (MQ/1) (CR/1) (EP/1) |
|  | Composite Carbon/Epoxy laminate |
|  | Composite Glass/Epoxy Laminate |
| Note Viaform Granular may be corrosive with galvanised steel. The corrosion rate of viaoform is low with approx 0.07mg/cm2 per day (compared to Na Cl at 0.2cm2 per day |  |

Environmental Data

Results:

1. Fish toxicity: 2.5

2. Bacterial toxicity: 1.2

3. Mammal toxicity: 1,0

Average: 1.6

The average value corresponds to the following Water Hazardous Classification:

Average value 			Water Hazardous

0 - 1.9 0 				Not hazardous to water

2.0 - 3.9 1 			Slightly hazardous to water

4.0 - 5.9 2 			Hazardous to water

6.0 3 				Strongly hazard

Biodegradability

Results:

BOD14  (mg O/l)      Lag phase(days)        Biodegradation(%) after 14 days

VIAFORM® LIQUID           		640               	5 			 90

Sodium Formate 			631 		5			 90

VIAFORM® LIQUID (potassium formate) and sodium formate are easily and totally biodegradable with very low oxygen consumption. Totally 90 % of the VIAFORM® LIQUID and sodium formate were biodegraded within 14 days.

According to data from the Huntingdon test, VIAFORM® LIQUID seems to have a slow initial degradation (the first 5 days). However, more than 60% of the VIAFORM® LIQUID is biodegraded within 10 days and 90% within 15 days.

Biodegradation studies performed at Jordforsk using VIAFORM® LIQUID showed a slightly prolonged lag phase, approx. 5 days. However, after 14 days more than 90% of VIAFORM® LIQUID was biodegraded.

Important factors:

* The total oxygen consumed during the degradation process is very low – Typically 1/3 compared with potassium acetate based products

* The end products is harmless (CO2 and Water)

* The product is totally and readily biodegradable.

Indicative application rates VIAFORM® LIQUID:

These recommended values can vary depending on elements like weather conditions, surface materials and spraying conditions.

| Frost or ice layer <1mm |  | Frost or ice layer 1-3mm |  |
| --- | --- | --- | --- |
| Temperature | Dosage | Temperature | Dosage |
| 0 to -5°C | 11 to 18 ml/m² | 0 to -5°C | 22 to 30 ml/m² |
| -5 to -10°C | 18ml to 26 ml/m² | -5 to -10°C | 30 to 44 ml/m² |

VIAFORM® LIQUID is available: -

- in 15 ltr Jerry can

- in 1000 ltr IBC containers

VIAFORM® LIQUID is delivered ready to use and must not be diluted due to the content of the corrosion inhibitor and the optimal melting capacity of VIAFORM® LIQUID at a concentration of 50 %.

VIAFORM® LIQUID can be stored in clean tanks made of polyethylene, fiberglass (approved for high pH) and painted or unpainted stainless steel. The use of galvanized steel in storage installations should be avoided due to the risk of corrosion.

VIAFORM® LIQUID is not classified as dangerous goods.